Literature Review

ML-Driven Adaptive Random Access Optimization for mMTC Smart Meter Networks

1. Introduction

The rise of smart grid infrastructure has turned Advanced Metering Infrastructure (AMI) into one of the most challenging use-cases for cellular IoT. A single NB-IoT cell in a dense urban area can serve tens of thousands of smart meters, each programmed to report consumption data at fixed 15-minute intervals under the IEC 62056 standard. Unlike generic IoT devices that transmit sporadically, smart meters are synchronised — a large fraction attempt RACH simultaneously, creating predictable yet severe congestion windows. Beyond routine reporting, demand-response events require up to 80% of meters to acknowledge a utility broadcast within two seconds, a hard operational SLA that standard 3GPP random access procedures were simply not designed to meet. The literature on random access optimization for massive machine-type communications (mMTC) is extensive, but as the following review shows, it has not yet produced a solution tailored to the specific constraints of smart metering — namely AMI-specific traffic patterns, joint multi-parameter control, guaranteed access delay, and deployment on resource-constrained hardware.

2. Review of Existing Literature

The analytical foundation for dynamic access barring was laid by Duan et al.[1], who proposed D-ACB for LTE M2M traffic and demonstrated around a 60% reduction in access delay over static schemes by estimating device load in real time. While effective, the model treated backoff timing and preamble count as fixed — a limitation that would prove consequential in later work. Leyva-Mayorga et al.[2] derived an optimal ACB factor analytically, showing that setting the barring probability proportional to available preambles maximises throughput; however, the model assumes instantaneous parameter updates and ignores backoff dynamics entirely. Together these works established that dynamic ACB matters, but also that the Backoff Indicator (BI) — which determines how long a device waits before retrying after a collision — had been left out of the optimisation picture completely.

The first credible evidence that lightweight reinforcement learning could work on constrained hardware came from Bello et al.[3], who showed tabular Q-learning with a discretised state space could improve RACH throughput by 55% and converge in under 200 episodes on a constrained device simulation. This was the signal that a low-complexity RL approach was not just theoretically possible but practically feasible — even if the work used generic Poisson traffic and did not extend to NB-IoT.

The shift to NB-IoT brought deeper complexity. Jiang et al.[4] proposed CMA-DQN, splitting a nine-parameter NB-IoT action space across cooperative agents and achieving a 6× improvement in access success under overload. The backoff indicator, however, was absent from the action space across all agents. Hadjadj-Aoul and Ait-Chellouche[5] applied TD3 to NB-IoT access control and improved successful preamble transmission by 42.5% over adaptive heuristics — but the continuous action space of TD3 makes on-device deployment impractical for M3/M4-class smart meter hardware. Abera et al.[6] and Bui and Pham[7] extended DQN-based ACB to LTE, demonstrating collision reduction of ~40% and energy savings of ~38% respectively, though both systems run cloud-side and treat delay as a soft metric rather than a hard constraint.

Traffic prediction emerged as a complementary direction. Lee et al.[8] showed an LSTM-based ACB scheme could cut mean access delay by 45% in LTE-A by predicting load ahead of time — yet accuracy fell to 72% during synchronised bursts, which is exactly the scenario that defines a demand-response event. Mehri et al.[9] extended this with a hybrid LSTM-DenseNet predictor (FLSP) achieving over 90% accuracy and 1–2 second ahead congestion forecasting, but the prediction output feeds no control mechanism for ACB, backoff, or preamble configuration. Prediction without a closed control loop leaves the congestion problem fundamentally unresolved.

Papers that directly modelled smart meter scenarios expose how serious the problem is under realistic conditions. Stusek et al.[10] simulated NB-IoT for 15-minute AMI reporting cycles and post-outage reconnection bursts, finding that even NB-IoT Release 15's Early Data Transmission feature could not contain the resulting collision storm, and explicitly called for ML-based adaptive control as future work. Litjens et al.[11] confirmed at the grid level that simultaneous outage-restoration access from all meters in a zone overwhelms standard RACH regardless of scheduling heuristics. Michelinakis et al.[12] measured real NB-IoT modems and found that 98.4% of ECL Level 2 transmissions operate at maximum TX power — a direct consequence of failed RACH attempts cascading into coverage enhancement mode and accelerated battery drain. Matz et al.[13] measured end-to-end latency on a live network and confirmed ECL Level 2 mean delay exceeds 10 seconds, making the 2-second demand-response SLA unachievable without active optimisation.

3. Identified Research Gaps

Reviewing this body of work, four clear gaps emerge. First, no paper models AMI-specific traffic: every study uses generic Beta or Poisson distributions that miss the deterministic 15-minute reporting cycle and the synchronised demand-response bursts that characterise real smart meter behaviour. Second, no work jointly optimises ACB, Backoff Indicator, and RACH configuration together: while ACB has received considerable attention and preamble allocation has been studied in isolation, the BI has been universally ignored across all reviewed papers — despite empirical evidence that unoptimised backoff is the primary driver of the RACH failure cascade into CE Level 2 escalation. Third, no existing approach enforces SLA as a hard constraint: delay is consistently treated as a metric to minimise rather than a boundary to guarantee, which is operationally inadequate when demand-response latency directly affects grid stability. Fourth, all ML solutions target base station or cloud deployment: smart meters typically run on M3/M4-class processors with limited flash memory and no floating-point unit. Silva and Abrão[14] showed that tabular Q-learning scales to 50,000 devices with a Q-table of a few kilobytes and O(1) lookup, and Bello et al.[3] confirmed on-device convergence — but neither applied this to NB-IoT, AMI traffic, or joint parameter control. Closing all four gaps in a single deployable system is the primary motivation for the proposed work.

Table 1: Comparison of Existing Approaches

(✓ = addressed,  ~ = partial,  ✗ = not addressed)

| Ref. | Method | NB-IoT | AMI Traffic | Joint ACB+BI+RACH | SLA Constraint | Embedded ML |
| --- | --- | --- | --- | --- | --- | --- |
| [1] | D-ACB (Analytical) | ✗ | ✗ | ✗ | ✗ | ✗ |
| [2] | Tabular Q-learning | ✗ | ✗ | ✗ | ✗ | ~ |
| [3] | Optimal ACB (Analytical) | ✓ | ✗ | ✗ | ✗ | ✗ |
| [4] | CMA-DQN (NB-IoT) | ✓ | ✗ | ~ | ✗ | ✗ |
| [5] | TD3 Deep RL | ✓ | ✗ | ✗ | ✗ | ✗ |
| [6] | DQN-ACB (LTE) | ✗ | ✗ | ✗ | ✗ | ✗ |
| [7] | DQN Energy+Delay | ✗ | ✗ | ✗ | ✗ | ✗ |
| [8] | LSTM-ACB | ✗ | ✗ | ✗ | ✗ | ✗ |
| [9] | LSTM+DenseNet (FLSP) | ✗ | ~ | ✗ | ✗ | ✗ |
| [10] | ns-3 Smart Meter Sim | ✓ | ✓ | ✗ | ✗ | ✗ |
| [11] | Grid Outage Sim | ✓ | ✓ | ✗ | ✗ | ✗ |
| [12] | Real HW (Energy) | ✓ | ✗ | ✗ | ~ | ✗ |
| [13] | Real HW (Latency) | ✓ | ✓ | ✗ | ~ | ✗ |
| Prop. | Tabular Q (Joint, AMI) | ✓ | ✓ | ✓ | ✓ | ✓ |

References

[1]  S. Duan, V. Shah-Mansouri, Z. Wang, and V. W. S. Wong, "D-ACB: Adaptive Congestion Control Algorithm for Bursty M2M Traffic in LTE Networks," IEEE Trans. Veh. Technol., vol. 65, no. 12, pp. 9875–9888, Dec. 2016.

[2]  I. Leyva-Mayorga et al., "Adaptive Access Class Barring for Efficient mMTC," Comput. Netw., vol. 149, pp. 86–99, 2019.

[3]  L. M. Bello, P. Mitchell, and D. Grace, "Q-Learning for Random Access in Wireless Networks," in Proc. 20th European Wireless Conf., Barcelona, 2014.

[4]  N. Jiang, Y. Deng, A. Nallanathan, and J. A. Chambers, "Reinforcement Learning for Real-Time Optimization in NB-IoT Networks," IEEE J. Sel. Areas Commun., vol. 37, no. 6, pp. 1424–1440, Jun. 2019.

[5]  Y. Hadjadj-Aoul and S. Ait-Chellouche, "Access Control in NB-IoT Networks: A Deep Reinforcement Learning Strategy," Information, vol. 11, no. 11, p. 541, 2020.

[6]  W. G. Abera, T. O. Olwal, and Y. W. Marye, "Deep Reinforcement Learning-Based Access Class Barring for mMTC," Array (Elsevier), vol. 28, p. 100533, 2025.

[7]  A. T. H. Bui and A. T. Pham, "DRL-Based ACB for Energy-Efficient mMTC Random Access in LTE," IEEE Access, vol. 8, 2020.

[8]  C.-H. Lee, S.-J. Kao, and F.-M. Chang, "LSTM-Based ACB Scheme for MTC in LTE-A Networks," Comput. Commun., vol. 152, pp. 192–200, Feb. 2020.

[9]  H. Mehri, H. Mehrpouyan, and H. Chen, "RACH Traffic Prediction in Massive Machine Type Communications," IEEE Trans. Mach. Learn. Commun. Netw., 2025.

[10] M. Stusek, J. Masek, P. Masek, and J. Hosek, "Exploiting NB-IoT Network Performance for Smart-Metering Use-Cases," in Proc. IEEE ICUMT, 2023.

[11] R. Litjens, A. Eisenblätter, and H. van den Berg, "Optimisation of NB-IoT Deployment for Smart Energy Distribution Networks," EURASIP J. Wireless Commun. Netw., 2019.

[12] F. Michelinakis et al., "Dissecting Energy Consumption of NB-IoT Devices Empirically," arXiv:2004.07127, 2020.

[13] M. Matz, M. Mraz, and J. Hosek, "A Systematic Analysis of Narrowband IoT Quality of Service," Sensors, vol. 20, no. 8, p. 2416, 2020.

[14] M. T. C. Silva and T. M. Abrão, "Throughput and Latency in Distributed Q-Learning Random Access for mMTC," Comput. Netw., vol. 215, 2022.