Machine Learning Approaches for Random Access Optimisation in Massive Machine-Type Communications

## Paper 1: Multi-Objective RACH Optimization via DRL and RNN

Source: 2005.01092v1.pdf

| Problem | Optimize RACH procedure in cellular mIoT for time-varying traffic and multi-objective performance encompassing success, delay, and energy. |
| --- | --- |
| ML/DL Method | DRL suite: Policy Gradient (PG), Actor-Critic (AC), DQN, DDPG; supplemented by RNN (GRU) for traffic prediction. |
| System Model & Math | Grant-based RACH modelled as POMDP. Hybrid reward: Rt = xs·Vst + xd·Rdt + xe·Ret (success component + delay component + energy component). RNN predicts current backlog Nt from observed preamble outcomes. |
| Setup & Dataset | N = 400 to 30,000 devices; 54 preambles; γmax = 10. Traffic: Beta(α=3, β=4) and Markovian inter-arrival patterns. |
| Tool | Not stated in source |
| Metrics | Success probability, average energy, average access delay, throughput. |
| Key Results | Decoupled learning yielded a 10x training speedup. DRL schemes significantly outperformed fixed ACB/BO methods under heavy traffic. |
| Traffic Handling | Bursty and emergency Beta traffic with synchronized patterns addressed via RNN-based prediction. |
| Limitations & Gaps | Base station relies on outdated delay and energy reports. Centralized architecture only — no cooperative learning. |

## Paper 2: RACH Traffic Forecasting via LSTM and DenseNet (FLSP)

Source: 2405.05235v2.pdf

| Problem | Forecast bursty traffic and detect congestion in multi-channel slotted ALOHA RACH for mMTC environments. |
| --- | --- |
| ML/DL Method | LSTM for sequence modelling, DenseNet for multi-scale feature extraction, and FFNN as a fast prediction head — combined as the Fast LiveStream Predictor (FLSP). |
| System Model & Math | Recursive multi-step traffic prediction. Congestion threshold: more than 7,000 collided attempts per interval. FLSP continuously updates LSTM hidden states for real-time inference. |
| Setup & Dataset | N = 60,000 devices; 54 preambles; L = 10 groups. Interval: 0.5 s. Traffic: Periodic/Uniform combined with event-driven Beta(α=3, β=4). |
| Tool | Python |
| Metrics | MSE, Precision, Recall, F1-score. |
| Key Results | FLSP achieved a 94.15% MSE improvement. Predicted congestion 1–2 seconds in advance with F1 = 0.92. |
| Traffic Handling | Periodic smart-meter readings combined with event-driven burst groups exhibiting distinct event probabilities. |
| Limitations & Gaps | Aggregated traffic only — no per-device source modelling. No integration with active ACB control mechanisms. |

## Paper 3: DQN-Based RACH for Heterogeneous QoS in LTE-M

Source: public version.pdf

| Problem | Regulate concurrent access and meet heterogeneous QoS requirements for mixed high- and low-priority traffic in LTE-M. |
| --- | --- |
| ML/DL Method | DQN with experience replay. Exploration strategies: Boltzmann softmax combined with ε-greedy. |
| System Model & Math | Markov Chain / state transition model. Weighted reward: r = 5·Nhigh_s + Nlow_s, which prioritizes delay-sensitive high-priority user equipment. |
| Setup & Dataset | Nhigh ≤ 15,000 devices (Beta distribution), Nlow ≤ 10,000 (Uniform). 54 preambles; 1 RAO every 10 ms. |
| Tool | TensorFlow (Python) |
| Metrics | Success probability, average end-to-end delay (EED), total success per RAO. |
| Key Results | DRL converged faster than tabular RL. Higher success probability for high-priority user equipment compared to the MACB baseline. |
| Traffic Handling | Synchronized and burst traffic for high-priority devices; periodic and uniform smart meter reporting for low-priority devices. |
| Limitations & Gaps | Centralized base station deployment only. UE-side deployment was deemed prohibitively costly due to processing constraints. |

## Paper 4: Tabular Q-Learning / DQN Comparison for NB-IoT RACH and NPUSCH

Source: 1812.09026v1.pdf

| Problem | Dynamically configure RACH (NPRACH) and data (NPUSCH) resources to maximise the number of served NB-IoT devices. |
| --- | --- |
| ML/DL Method | Tabular Q-learning, LA-Q, and DQN. Advanced variants: AA-LA-Q and AA-DQN (action aggregation). Best performer: CMA-DQN (cooperative multi-agent DQN). |
| System Model & Math | Formulated as a POMDP. Reward: number of successfully served devices. State: {collided preambles, successful preambles, idle preambles, waiting devices}. |
| Setup & Dataset | N = 10,000–30,000 periodic devices + 5,000–30,000 bursty devices. 12–48 preambles. Traffic: Uniform combined with Beta. |
| Tool | Python (TensorFlow) |
| Metrics | Served devices (Vsu), average reward, training time, dropped packets (Vdrop). |
| Key Results | CMA-DQN achieved the best throughput and efficiency. DQN optimised long-term packet drops more effectively than one-step methods. |
| Traffic Handling | Heterogeneous mixture: periodic uniform reporting combined with synchronized, event-driven Beta bursts. |
| Limitations & Gaps | Standard Q-learning fails in high-dimensional spaces, justifying the action aggregation and multi-agent architectures introduced. |

## Paper 5: CPCL — Coupled Prediction and Control Learning for RA Overload

Source: 2002.07759v1.pdf

| Problem | Joint traffic prediction and RA control to handle massive access overload in cellular networks. |
| --- | --- |
| ML/DL Method | CPCL: RNN prediction module coupled with DRL control module. Bayesian probability inference employed for traffic prediction. |
| System Model & Math | POMDP on f-ALOHA structure. Prediction uses Bayesian inference of future traffic load. Control module receives the predicted load as an additional state input. |
| Setup & Dataset | 54 channels; γmax = 10. Traffic: Time-limited Beta(3,4) profile repeated every 10 frames. |
| Tool | Python |
| Metrics | Success probability, average successful devices per episode, convergence speed. |
| Key Results | CPCL converged approximately 100 times faster than conventional DQN. Demonstrated superior performance relative to ACB and ACB+BO baselines. |
| Traffic Handling | Periodic, event-driven, and streaming patterns captured via temporal RNN correlations. |
| Limitations & Gaps | One-step RL suffers from low efficiency; traditional methods are inaccurate due to oversimplified traffic assumptions. |

## Paper 6: Dueling DQN for Energy-Efficient ACB in LTE

Source: Deep_Reinforcement_Learning-Based_Access_Class_Barring.pdf

| Problem | Energy-efficient ACB optimisation balancing access delay and energy consumption under bursty access scenarios. |
| --- | --- |
| ML/DL Method | Dueling DQN. State: {successes, collisions, delay, energy}. Action: combined (pACB, TACB) pair. |
| System Model & Math | MDP formulation. Reward: weighted satisfaction of delay and energy thresholds. Tunes both barring factor pACB and mean barring time TACB jointly. |
| Setup & Dataset | 54 preambles. Traffic: Bursty Beta(3,4). Dthresh = 1000 subframes; Tarr = 10 s. |
| Tool | Not stated in source |
| Metrics | Delay satisfaction, energy satisfaction, average delay, blocking ratio. |
| Key Results | 16% improvement in energy satisfaction. 25–42% energy reduction compared to schemes that tune pACB alone. |
| Traffic Handling | Highly synchronized and bursty traffic — dynamically tunes both barring factor and mean barring time. |
| Limitations & Gaps | Previous solutions focused solely on pACB, resulting in high energy consumption during continuous downlink monitoring. |

## Paper 7: LSTM for Resource Slicing in Mixed URLLC-mMTC

Source: LSTM-ACB-Based_Random_Access_for_Mixed_Traffic_IoT.pdf

| Problem | Random access optimisation in mixed URLLC and mMTC scenarios ensuring QoS compliance and efficient resource utilisation. |
| --- | --- |
| ML/DL Method | LSTM neural network for backlog estimation and resource slicing decisions. |
| System Model & Math | M-MIMO with Orthogonal Multiple Access (OMA). Backlog estimation via Bayesian inference. Resource slicing based on 5G NR numerology. |
| Setup & Dataset | 25 URLLC + 1,000 mMTC devices; 9 MHz bandwidth; 54 preambles. Traffic: Periodic mMTC combined with Beta-distributed URLLC. |
| Tool | Monte-Carlo simulation |
| Metrics | Normalised throughput, channel loading (CL), MSE for backlog prediction. |
| Key Results | LSTM MSE improved from 10^-2 to 10^-6. Resource slicing increased channel allocation from 30 to 41 channels. |
| Traffic Handling | Heterogeneous mixture: periodic mMTC arrivals combined with event-driven URLLC Beta traffic. |
| Limitations & Gaps | Grant-free benchmarks become overloaded under high congestion. No joint optimisation of prediction and slicing. |

## Paper 8: TD3 for Access Control and Congestion Management in NB-IoT

Source: information-11-00541-v2.pdf

| Problem | Access control and congestion management in NB-IoT to address massive connectivity challenges. |
| --- | --- |
| ML/DL Method | TD3 (Twin Delayed Deep Deterministic Policy Gradient). Continuous action space with dual critic networks and delayed policy update. |
| System Model & Math | NPRACH modelled as a 4-step RA process. MDP formulation. Fluid model for terminal dynamics with states: {backlogged, blocked, attempting}. Measurement horizon H = 10. |
| Setup & Dataset | N = 12 preambles; Poisson arrival (λ = 0.018 s^-1); maximum attempts: 16. |
| Tool | SimPy (Python) |
| Metrics | Average access latency, average reward, successful preambles, abandonments. |
| Key Results | TD3 achieved 3.52 successful preambles versus the theoretical optimum of 4.61 — a 42.51% improvement over adaptive controllers. |
| Traffic Handling | Complex mixture of Poisson, Uniform, and Beta distributions — robust under traffic uncertainty. |
| Limitations & Gaps | Network state is unobservable with noisy measurements. PID and adaptive controllers fail on non-linear inputs. |

- Research Gap Analysis :

| Paper | ML/DL Method | NB-IoT | AMI Traffic | Joint Opt. | BI Tuned | Hard SLA | Embedded | Real Data |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [1] | DRL (PG/AC/DQN/DDPG) + RNN | No | No | Partial | No | No | No | No |
| [2] | LSTM + DenseNet + FFNN | No | Partial | No | No | No | No | No |
| [3] | DQN (Boltzmann + ε-greedy) | No | Partial | No | No | Partial | No | No |
| [4] | Tabular Q / LA-Q / CMA-DQN | Yes | No | Partial | No | No | Partial | No |
| [5] | RNN + DRL (CPCL) | No | No | Partial | No | No | No | No |
| [6] | Dueling DQN | No | No | Partial | Yes | No | No | No |
| [7] | LSTM | No | Partial | No | No | Partial | No | No |
| [8] | TD3 (Continuous DRL) | Yes | No | No | No | No | No | No |
| PROPOSED | Tabular Q-Learning (Joint) | Yes | Yes | Yes | Yes | Yes | Yes | AMI Sim |